|  | NOTES PERTAINING TO ANY DEFERRALS PER THIS LISTING: - PRIOR TO ITEM DEFERRAL, THE FAA APPROVED MEL AND MMEL MUST BE CHECKED TO ASSURE THAT THE INDIVIDUAL ITEM(S) TO BE DEFERRED ARE NOT LISTED IN EITHER. - The electrical connector supplying power to any electrical device removed per this listing must be secured as required and protected from the possibility of electrical short and/or arching. This is a maintenance activity to be performed by certified maintenance personnel. - A DMI placard must be placed on any inoperative or damaged equipment or in the location of any missing equipment that is normally affixed to the aircraft. For Gulfstream aircraft (GIV, GV, GV-X, GV-SP, G500), the repair category is limited to category “D”. |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | GENERAL |  |  |  |
|  | 1) | Placards - Interior, Required - Interior, Not Required - Exterior, Required - Exterior, Not Required | E | May be: - Degraded (but legible) - Degraded, Missing, or Illegible - Degraded (but legible) - Degraded, Missing, or Illegible |
|  | 2) | Paint Defects / Decorative Finishes | E | Exterior paint may be worn, damaged, or missing Decorative plating may be missing or damaged |
|  | 3) | Sealant, Aerodynamic and Chafe | E | May be degraded, cracked or crazed |
|  | 4) | Cabinets, Closets, Credenzas, Vanities, Drink Rails, etc. | E | Drawers, Compartment doors, Shelving and latches may be inoperative provided the associated compartment is empty and secured in the closed position. |
|  | 5) | Cup Holders | E | May be inoperative, damaged or missing |
|  | 6) | Crew / Passenger Seat Cushions, Upholstery | E | May be worn, torn, frayed or stained as long as the item is otherwise serviceable |
|  | 7) | Gaspers / Air Vents / Ventilation Grilles | E | May be inoperative or damaged provided airflow is acceptable |
|  | 8) | Drawer Latches / Drawer Locks / Drawer Slides | E | May be inoperative provided the associated drawer is empty and secured in the closed position. |
|  | 9) | Coat Rods, Hangers, Hooks, Curtains, and Clips | E | May be inoperative, damaged or missing |
|  | 10) | Appearance Items - Cabin Woodwork and Trim - Decorative Trim / Trim Strips - Carpet / Floor Coverings / Threshold Plates / Floor Liner - Curtains / Tiebacks / Snaps / Straps - Wall Coverings (including sidewall panels, excluding sidewall return air grilles) - Trim - Seat, Sidewall, Overhead, etc. - Headliner / PSU Covering / Upholstery - Seatbelt Tidy Clips - Window or Windshield Protective Cover - Stairs – Tread / Decorative Coverings | E | May be Worn, Soiled, Frayed, Torn, Damaged, Loose or Missing (may not present hazard to passenger / crew or impede emergency egress / or needed to retain another cabin item) |
|  | 11) | Vent Knobs | E | May be inoperative, damaged or missing provided airflow is acceptable |

| 1. | GENERAL (cont’d) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 12) | 115 VAC Outlets and Covers | E | May be inoperative or missing. The covers may be damaged or missing (M)(C) Procedure – Maintenance will electrically isolate the affected outlet by pulling and collaring the appropriate circuit breaker or (Maintenance only) by capping and stowing the affected wires to disable outlet. If cover is damaged or missing, no maintenance action is required. |
|  | 13) | Flashlight Chargers / Flashlight Clips | E | May be inoperative or missing |
|  | 14) | Sinks / Drains / Faucets / Screens | E | May be inoperative or missing |
|  | 15) | Cabin Lighting / Signs (Only items not covered by MEL/MMEL ATA 33) (Excludes any emergency lights) - Reading Lights - Table lights - Indirect / Overhead Lights - Floor Illumination Lights - Sign Light Lens / Covers (including emergency light) - Lavatory Occupied Lights - Service Lights - Over Wing Exits Unlocked Indications (G500) | E | May be inoperative or missing NOTE: Sign Lights / Lens / Lens Grill Assembly may be loose or damaged but light must function properly. |
|  | 16) | Crash Axe | E | May be missing |
|  | 17) | Security Boxes / Safe / Lock Box | E | May be inoperative, damaged or missing |
|  | 18) | Insulation Blankets | E | May be missing |
|  | 19) | Work Tables | E | May be inoperative or missing |
|  | 20) | Universal Precautions Kit (Bloodborne Pathogen Kit) | E | May be inoperative, damaged or missing |
|  | 21) | A/C Water System / Components (Only if not covered by MEL/MMEL ATA 38) | E | May be inoperative or missing |
|  | 22) | Airworthiness Certificate / Registration | E | May be deteriorated as long as it is readable |
|  | 23) | Document Holder for item 22 above | E | May be damaged or deteriorated |
|  | 24) | Defibrillator Pads | E | May be out of shelf life, damaged or missing |
|  | 25) | Switch Capsule Lighting | E | May be inoperative provided those switch capsules control passenger convenience items only. |
|  | 26) | A/C polished exterior surfaces (leading edges, engine inlets, slats, scoops, etc.) | E | May have minor scratching, abrasion or tooling marks that are cosmetic in nature only. |
|  | 27) | USB Port Inoperative | E | May be inoperative or missing |
|  | 28) | Passenger Phone System (or individual units) | D | May be inoperative or missing |
|  | 29) | Passenger Audio Entertainment System - Jack(s) - Passenger Seat Volume Control - Speaker(s) - System(s) | D | May be inoperative or missing |
|  | 30) | Passenger Seat Call - Passenger Seat Call Button(s) - Passenger Seat Call Light(s) - VIP panel | D | May be inoperative or missing |

| 2. | COCKPIT |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1) | Sunshades | E | May be inoperative, damaged or missing |
|  | 2) | Smoke Goggle Stowage Bags | E | May be inoperative, damaged or missing |
|  | 3) | Side Panel Map Cases | E | May be inoperative, damaged or missing provided crew has an alternate location for map storage |
|  | 4) | Side View Mirrors | E | May be inoperative, damaged or missing |
|  | 5) | Foot Rests | E | May be inoperative, damaged or missing |
|  | 6) | Foot Warmer Handle and Vents | E | May be inoperative, damaged or missing as long as the airflow through the foot warmer is acceptable to the flight crew. |
|  | 7) | Chart Holder / Yoke Clips | E | May be inoperative, damaged or missing |
|  | 8) | Hand Microphone Holders / Headset Holders | E | May be inoperative, damaged or missing |
|  | 9) | Storage/Access Compartment Latches | E | May be inoperative, damaged or missing |
|  | 10) | Ashtrays | E | May be inoperative, damaged or missing |
|  | 11) | Document Holders (Vibration Card / Compass Card) | E | May be inoperative, damaged or missing provided documentation is readily available to the flight crew. |
|  | 12) | Spring Clips (incl. Chart Holders) | E | May be inoperative, damaged or missing |
|  | 13) | HUD Protective Cover | E | May be inoperative, damaged or missing |
|  | 14) | Spare Bulb Kit and Contents | E | May be inoperative, damaged or missing |
|  | 15) | Windshield / Side Window Heat Contact Covers | E | May be deteriorated provided functionality is not affected. |
|  | 16) | Vanity Door Mirror | E | May be inoperative, damaged or missing |
|  | 17) | Food Trays | E | May be inoperative, damaged or missing |
|  | 18) | Foot Tread Trim | E | May be inoperative, damaged or missing |
|  | 19) | Logbook Holder | E | May be inoperative, damaged or missing |
|  | 20) | Observers Foot Rest / Foot Holder | E | May be inoperative, damaged or missing |
|  | 21) | Spare Headsets | E | May be inoperative, damaged or missing |
|  | 22) | Note Pads and Holders | E | May be inoperative, damaged or missing |
|  | 23) | Pens / Pencil Holders | E | May be inoperative, damaged or missing |
|  | 24) | Side Window Pull out Shades | E | May be inoperative, damaged or missing |
|  | 25) | Electronic Charts out of Currency Note: This item only applies to aircraft that do not have this provision in the FAA Approved Minimum Equipment List. | E | May be out of currency provided current aeronautical charts are used to verify navigation fixes before dispatch - Procedure – Verify revision status of charts required to conduct the flight are current and have not expired. Check for updates. |
|  | 26) | Navigation Database out of Currency Note: This item only applies to aircraft that do not have this provision in the FAA Approved Minimum Equipment List. | E | May be inoperative provided current aeronautical charts are used to verify navigation fixes before dispatch - Procedure – Verify revision status of charts required to conduct the flight are current and have not expired. Check for updates. |
|  | 27) | WSI Weather System / Components Note: This item only applies to aircraft that do not have this provision in the FAA Approved Minimum Equipment List. | E | May be inoperative or missing (M)(C) Procedure – Pull and Collar WSI Circuit Breaker |
|  | 28) | Day / Night Mode on Switch Capsule Inoperative | E | May be inoperative provided switch capsule functions normally |
|  | 29) | Glare Shield Handhold | E | May be inoperative, damaged or missing |
|  | 30) | Cockpit Annunciator Lighting Bezels and Lenses | E | Bezels may be damaged or missing, lenses on non-worded switch capsules. Note: Lighting must function properly |
|  | 31) | Audio Control Panel | D | May be Inoperative or missing Note: Special placard for DDL cover of AFL needed stating: “Jump seat may not be used for FAA observer with audio panel deferred”. |

| 2. | COCKPIT (cont’d) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 32) | EFB Power Switch Indication Light | D | May be inoperative provided switch functions normally |
|  | 33) | Observer’s Storage Compartment | D | May be inoperative or missing |
|  | 34) | Step Well Storage Box and Latch | D | May be inoperative or missing (M)(C) Procedure - Maintenance will secure cover closed. |
| 3. | GALLEY |  |  |  |
|  | 1) | Coffee Maker | E | May be inoperative, damaged or missing |
|  | 2) | Coffee / Tea / Hot Pots / Thermos Units | E | May be inoperative, damaged or missing |
|  | 3) | Convection Oven and Controls | E | May be inoperative, damaged or missing |
|  | 4) | Microwave Oven and Controls | E | May be inoperative, damaged or missing |
|  | 5) | Hot Cup and Controls | E | May be inoperative, damaged or missing |
|  | 6) | Water Heater | E | May be inoperative |
|  | 7) | Waste Containers | E | May be inoperative, damaged or missing |
|  | 8) | Espresso Machine and Controls | E | May be inoperative or missing |
|  | 9) | Ice Container / Ice Chest / Ice Drawer / Chillers / Cooler Units / Refrigeration System | E | May be inoperative, damaged or missing |
|  | 10) | Warming Plates / Warming Units /Warmer Pads | E | May be inoperative, damaged or missing |
|  | 11) | Pots, Pan, and Utensils | E | May be inoperative, damaged or missing |
|  | 12) | Catering Containers / Plates | E | May be inoperative, damaged or missing |
|  | 13) | Water Boilers / Hot Water Dispenser | E | May be inoperative or missing |
|  | 14) | Water System | E | May be inoperative (Verify that MEL/MMEL does not address this item) |
|  | 15) | Plate Warmers | E | May be inoperative, damaged or missing |
|  | 16) | Beverage / Cup Dispenser | E | May be inoperative, damaged or missing |
|  | 17) | Galley Heating and Controls | E | May be inoperative, damaged or missing |
|  | 18) | Work Tables / Cutting Boards | E | May be inoperative, damaged or missing |
|  | 19) | Bun Warmer | E | May be inoperative, damaged or missing |
|  | 20) | Galley Floor Heater | E | May be inoperative, damaged or missing |
|  | 21) | Paper Clips | E | May be inoperative, damaged or missing |
|  | 22) | Chilled Air Temperature Gauge | E | May be inoperative, damaged or missing |
|  | 23) | Mirrors | E | May be inoperative, damaged or missing |
|  | 24) | Flashlight / Flashlight Charger Unit | E | May be inoperative, damaged or missing |
|  | 25) | Liquor Bottle Holders / Wine Racks | E | May be inoperative, damaged or missing |
|  | 26) | Paper Towel Racks | E | May be inoperative, damaged or missing |
|  | 27) | Lock Post for Galley Doors / Drawers | E | May be inoperative, damaged or missing |
|  | 28) | Water Level Indicator | E | May be inoperative, damaged or missing |
|  | 29) | Work Lights | E | May be inoperative, damaged or missing |
|  | 30) | Galley Doors / Tambour Doors | E | May be inoperative or missing provided installed equipment contained by doors is secured or removed from cabinet. |
|  | 31) | Galley Power Switch | E | May be inoperative (M)(C) Procedure - Pull and Collar Galley Power C/B Placard must be placed on the DDL Section Cover in the AFL ring binder listing all galley items that will be inoperative. |
|  | 32) | Galley Cart (Trolley) | E | May be inoperative, damaged or missing (M) Procedure - Update airplane weight and balance manual |

| 3. | GALLEY (cont’d) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 33) | Galley Master Switch Panel | D | May be inoperative |
|  | 34) | Coat Hanger Clips | D | May be inoperative, damaged, or missing |
| 4. | CABIN |  |  |  |
|  | 1) | Acoustic Door/ Cover / Curtains | E | May be inoperative, damaged or missing (M)(C) Procedure – Maintenance must secure door in open position. |
|  | 2) | Spare Life Vests | E | May be missing |
|  | 3) | Magazine Rack / Restraint | E | May be inoperative, damaged or missing |
|  | 4) | Bulkhead /Individual Video Screens | E | May be inoperative, damaged or missing |
|  | 5) | Stereo Equipment | E | May be inoperative, damaged or missing |
|  | 6) | Individual Video Players/Screens/Monitors | E | May be inoperative, damaged or missing |
|  | 7) | Entertainment Controllers | E | May be inoperative, damaged or missing |
|  | 8) | Wireless / Wired LAN | E | May be inoperative, damaged or missing |
|  | 9) | High Speed Data Link Communication System | E | May be inoperative, damaged or missing |
|  | 10) | Ash Trays (Must be in compliance with AD 74-08-09 as revised) | E | May be inoperative or missing |
|  | 11) | Headphones / Headphone Outlets | E | May be inoperative, damaged or missing |
|  | 12) | Under Carpet Heater Pads | E | May be inoperative, damaged or missing (M)(C) Procedure – Maintenance will pull and collar affected circuit breakers. |
|  | 13) | Movie Projectors (includes VCR’s, DVD Players, Video Games) | E | May be inoperative, damaged or missing |
|  | 14) | Acoustic Doors / Curtains / Curtain Hooks / Curtain Rail | E | May be inoperative or missing (M)(C) Procedure – Maintenance must secure door in open position. |
|  | 15) | Cabin Mirrors | E | May be damaged or missing |
|  | 16) | Seat Track Covers | E | May be inoperative, damaged or missing |
|  | 17) | Cabin Windows | E | May Be: - Crazed (if within maintenance manual limits) or have Moisture Between Panes - Interior Panel may be Damaged or Need Cleaning - Shades may be damaged or Inoperative - Window Trim may be Loose, Damaged or Missing |
|  | 18) | Table Lift Mechanisms | E | May have lift mechanisms inoperative provided the table is set and secured in the appropriate position for takeoff, approach, and landing. NOTE: If table cannot be secured in the appropriate position, then the seats associated with that table must be considered as inoperative (refer to MEL) |
|  | 19) | Fire Extinguisher Seal | E | May be inoperative or missing provided the safety pin is intact and otherwise fully serviceable |
|  | 20) | Cabin / Galley Handset Cradle | E | May be inoperative, damaged or missing provided the handset is fully operational |
|  | 21) | Therapeutic Oxygen Fittings | E | May be inoperative, damaged or missing |
|  | 22) | Universal Precaution Kit | E | May be missing or contents used |
|  | 23) | Cabin Divider Panels / Cabin Doors | E | May be difficult to close but must operate |
|  | 24) | Attendant Work Table | E | May be inoperative, damaged or missing |
|  | 25) | Attendant Storage Box | E | May be inoperative, damaged or missing |

| 4. | CABIN (cont’d) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 26) | Airfone / Flightphone | E | Individual Handsets may be inoperative, damaged or missing |
|  | 27) | Cabin Speakers | E | May have static when used, but must be operational |
|  | 28) | Emergency Floor Light Cover | E | May be damaged |
|  | 29) | Therapeutic Oxygen Mounting Anchor | E | May be missing |
|  | 30) | Seatback / Bulkhead / Sidewall Literature Pockets | E | May be inoperative, damaged or missing |
|  | 31) | Aircraft Removable Equipment | E | May be inoperative, damaged or missing |
|  | 32) | Non-Permanent Table Pads | E | May be soiled, frayed or missing Note: May only be missing if not a safety related pad required by placarding (M) Procedure – Attachment Bracket may be missing provided alternate means of attachment (such as tape) is made. |
|  | 33) | Facsimile (Fax) Machine | E | May be inoperative, damaged or missing |
|  | 34) | Flashlight | E | May be inoperative or missing |
|  | 35) | Passenger Seat Footrest and/or Forward Facing Passenger Seat Headrest | E | May be inoperative, missing, damaged, torn, soiled or frayed. |
|  | 36) | Airshow System | E | May be inoperative or missing |
|  | 37) | Printer | E | May be inoperative, damaged or missing |
|  | 38) | Latch (Cabin Door Hold Closed) | E | May be inoperative or missing |
|  | 39) | Under Carpet Floorboard Shim Panels | E | May be damaged (Applies to Falcon 2000LXS Only) |
|  | 40) | Forward Cabin Pocket Door | E | May be inoperative or missing (M)(C) Procedure – Secure door closed with Tape |
|  | 41) | Cabin Interior Window Heating Element Panes (Cracks) | D | May be cracked with no limitation on cabin pressurization as pane is not a structural part of window. |
|  | 42) | Miscellaneous Cabin Forms | D | May be missing or needs replenishing |
|  | 43) | Safety Demo Equipment - Demo Tape / CD (notify cabin attendant, physical demo may be required.) - Demo Equipment: Safety belt, life vest, O2 mask (consider availability of safety video) | D | May be inoperative or missing (G500) |
|  | 44) | Fixed Cabin Pedestal Tale | D | May be inoperative, damaged or missing (M)(C) Procedure – Maintenance will ensure table is secured in the “Up” position. |
|  | 45) | Folding Cabin Console Tables | D | May be inoperative, damaged or missing (M)(C) Procedure – Maintenance will ensure table is secured in the “Stowed” position. |
| 5. | FORWARD / AFT LAVATORIES |  |  |  |
|  | 1) | Door Spring | E | May be inoperative, damaged or missing |
|  | 2) | Sanitizer Dispenser | E | May be inoperative, damaged or missing |
|  | 3) | Seal Strips on Lavatory Door | E | May be inoperative, damaged or missing |
|  | 4) | Privacy Stop on Lavatory Door | E | May be inoperative, damaged or missing |
|  | 5) | Vanity Mirror and/or Vanity Lights | E | May be inoperative, damaged or missing |
|  | 6) | Lavatory Seat Cover Holder | E | May be inoperative, damaged or missing |
|  | 7) | Toilet Paper / Roller | E | May be inoperative, damaged or missing |
|  | 8) | Lights (Excluding Emergency Lighting) | E | May be inoperative, damaged or missing |
|  | 9) | Mirror Lights | E | May be inoperative, damaged or missing |
|  | 10) | Sinks / Drains / Faucets / Handles | E | May be inoperative, damaged or missing |

| 5. | FORWARD / AFT LAVATORIES (cont’d) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 11) | Assist Handles | E | May be inoperative, damaged or missing |
|  | 12) | Toilet Seat Lid | E | May be inoperative, damaged or missing |
|  | 13) | Toilet Seat | E | May be inoperative, damaged or missing |
|  | 14) | Toilet / Porta-potty | E | May be inoperative, damaged or missing |
|  | 15) | Soap Holder / Soap Dispenser | E | May be inoperative, damaged or missing |
|  | 16) | Hot and Cold Faucet Indicators | E | May be inoperative, damaged or missing |
|  | 17) | Deodorizer Holder | E | May be inoperative, damaged or missing |
|  | 18) | Bio Hazard Disposal Container | E | May be inoperative, damaged or missing |
|  | 19) | Paper Towels / Holder | E | May be inoperative, damaged or missing |
|  | 20) | Air Grill Covers | E | May be inoperative, damaged or missing |
|  | 21) | Tissue Dispenser | E | May be inoperative, damaged or missing |
|  | 22) | Pressure Pad | E | May be inoperative, damaged or missing |
|  | 23) | Towel rods / Towels | E | May be inoperative, damaged or missing |
|  | 24) | Water System / Water Heater | E | May be inoperative, damaged or missing |
|  | 25) | Waste Container | E | May be inoperative, damaged or missing |
|  | 26) | Flashlight | E | May be inoperative or missing |
|  | 27) | Lavatory Lighting Controls | E | May be inoperative or missing (M)(C) Procedure – electrically isolate lighting control by pulling and collaring the appropriate circuit breaker or (Maintenance only) by capping and stowing the affected wires to disable lighting control. |
|  | 28) | Faucet Aerator | D | May be inoperative or missing |
|  | 29) | Inside Door Ashtrays Note: Exterior Lav. Door Ashtrays are not considered NEF items. | D | May be inoperative or missing |
|  | 30) | Lights | D | May be inoperative or missing |
|  | 31) | Paper Towels / Holder | D | May be inoperative or missing |
|  | 32) | Sink Drain Screen | D | May be inoperative or missing |
|  | 33) | Toilet | D | May be inoperative or missing |
| 6. | FORWARD / AFT CLOSETS |  |  |  |
|  | 1) | Closet Doors / Curtains | E | May be inoperative, damaged or missing |
|  | 2) | Manual Storage Compartments | E | May be inoperative, damaged or missing |
|  | 3) | Flashlight | E | May be inoperative, damaged or missing |
|  | 4) | Umbrellas | D | May be inoperative or missing |
| 7. | VESTIBULE |  |  |  |
|  | 1) | Crew Name Plaque | E | May be inoperative, damaged or missing |
|  | 2) | Overhead Lights (Excluding Emergency Lights) | E | May be inoperative, damaged or missing |
|  | 3) | Radio Rack Decorative Closeouts | D | May be inoperative, damaged, or missing |
| 8. | CREW REST AREAS |  |  |  |
|  | 1) | Air Vents / Grills | E | May be inoperative, damaged or missing |
|  | 2) | Phone Holder | E | May be inoperative, damaged or missing |
|  | 3) | Stowage Compartment and Lights (Excluding Emergency Lighting) | E | May be inoperative, damaged or missing |
|  | 4) | Flashlight | E | May be inoperative or missing |
|  | 5) | Door | E | May be inoperative (M)(C) Procedure – If required, Maintenance will secure the affected door closed. Flight Crew may perform if maintenance is not available. |
|  | 6) | Mirror | E | May be damaged or missing (may not pose a safety hazard to passengers or crew) |

| 8. | CREW REST AREAS (cont’d) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 7) | Ash Trays | E | May be inoperative, damaged or missing |
|  | 8) | Assist Handles | E | May be inoperative, damaged or missing |
|  | 9) | Floor Track Covers | E | May be inoperative, damaged or missing |
|  | 10) | Video Screen | E | May be inoperative, damaged or missing |
|  | 11) | Door Latches | D | May be inoperative or missing |
|  | 12) | Pillow / Blankets / Sheets | D | May be inoperative or missing |
|  | 13) | Crew Rest Area Seat / Bed | D | May be inoperative or missing including the seatbelt, shoulder harness (if installed), recline mechanism, berthing mechanism, armrests, swivel mechanism, oxygen mask, and electrical/electronic systems/components. - Procedure – If required, Maintenance will secure the seat/bed in a position acceptable to the crew member(s) who will occupy the crew rest area. - Procedure – Flight crew will review the seat/bed to be acceptable for use as a resting place when not acting as a cockpit crew member. If not acceptable, flight crew will occupy other available seat in the cabin. |
| 9. | AFT BAGGAGE COMPARTMENT |  |  |  |
|  | 1) | Lights / Lighting System (Excluding Emergency Lighting) | E | May be inoperative, damaged or missing |
|  | 2) | Flashlight | E | May be inoperative, damaged or missing |
|  | 3) | Baggage Door Cable / Reel Assembly | E | May be inoperative, damaged, or missing |
|  | 4) | Fly Away Kit / Bag (including contents) | E | May be inoperative, damaged, or missing |
|  | 5) | Exterior Door Protective Mat | D | May be inoperative or missing |
| 10. | EXTERNAL SERVICE BAYS |  |  |  |
|  | 1) | Includes the following External Service Bays: (Items Not Covered by MMEL ATA Chapter 33 Only) - External Power Service Bay - Oxygen Service Bay - Toilet Service Bay - Potable Water Service Bay - Hydraulic Service Bay - Ground Refueling Service Bay - Tail Compartment Service Bay - Ground Air Service Bay - Wheel Well Lights - Aft Fuselage Service Bay | E | External Service Bay(s) may: - Have inoperative, damaged or missing Hold Open Rods or Lanyards - Have lights / lenses that are inoperative, damaged or missing - Have lighting switches that are inoperative or missing - Have damaged or missing dust covers and dust cover lanyards/chains/caps - Have Water Quantity Indication that is inoperative or missing (G500) - Hydraulic Service Bay manual service hoses may be inoperative or missing (G500) |
|  | 2) | Antenna / Drainmast Chafe Tape | E | May be deteriorated or missing |
|  | 3) | Aerodynamic Seals (Thrust Reverser to Pylon / Gear Door / Flap Well | E | May be deteriorated only. May not be missing |
| 11. | AIRCRAFT EXTERIOR |  |  |  |
|  | 1) | Engine Fire Bottle Lenses | E | May be crazed |
|  | 2) | Gear Pin Flags | E | May be inoperative, damaged or missing |
|  | 3) | Grease Fittings | C | May be inoperative or missing |
|  | 4) | Entrance Door Rain Gutter | E | May be deteriorated only. May not be missing |

| 11. | AIRCRAFT EXTERIOR (cont’d) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5) | Engine / Pitot / APU Intake Covers | E | May be inoperative, damaged or missing |
|  | 6) | Aircraft Tail Stands | E | May be inoperative, damaged or missing |
|  | 7) | Radome Chafe Boot | C | May be deteriorated or missing |
|  | 8) | Tire Pressure Sensor Valve (Tire Mounted) | E | May be inoperative or missing |
|  | 9) | Water Drain Mast Heater | E | May be inoperative (M)(C) Procedure - Disable the water drain mast heater by pulling and collaring the circuit breaker. |